# INVERSE POWER FLOW SYSTEM

# USER MANUAL

Version 1.0 | 2026-04-25

# 1. INTRODUCTION

## 1.1 Purpose

The Inverse Power Flow System is a modular Python-based tool for power system analysis. Unlike traditional power flow solvers that iterate to find voltages from given powers, this system:

• Takes voltage profiles as input

• Calculates the resulting power flows and injections

• Generates multiple random samples for statistical analysis

• Produces comprehensive reports in multiple formats

## 1.2 System Components

| File | Function | Modifiable? |
| --- | --- | --- |
| frompypowerformatetothis.py | Converts PYPOWER files to custom format | Yes |
| input_reader.py | Reads and parses input files | Yes |
| power_flow_engine.py | Core calculation engine | NO |
| report_writer.py | Generates output reports | Yes |
| run_power_flow.py | Main execution script | Yes |

# 2. INPUT SPECIFICATIONS

## 2.1 Input File Format

The system accepts Python (.py) files with specific variable names:

Required Variables:
━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━
BASE_MVA = 100                    # System base MVA

BUS_DATA = [                   # Bus information
    [bus_num, name, type, base_kV, V_init, angle, G, B, area, zone, owner],
    ...
]

GENERATOR_DATA = [              # Generator information
    [gen_num, name, type, bus, P_out, Q_out, V_set, Qmin, Qmax, status, area, zone],
    ...
]

LOAD_DATA = [                   # Load information
    [load_num, name, bus, P_demand, Q_demand, model, area, zone],
    ...
]

LINE_DATA = [                   # Transmission line information
    [line_num, name, from_bus, to_bus, length_km, R_per_km, X_per_km, 
     B_per_km, rateA_MVA, status, area, zone, owner],
    ...
]

TRANSFORMER_DATA = [            # Transformer information
    [xfmr_num, name, from_bus, to_bus, R_pu, X_pu, tap_ratio, rateA_MVA,
     phase_shift, min_tap, max_tap, step_size, status, area, zone, owner],
    ...
]

# Optional (can be empty lists):
CAPACITOR_DATA = []
REACTOR_DATA = []
SERIES_COMP_DATA = []
SERIES_REACTOR_DATA = []
SHUNT_DATA = []

## 2.2 Bus Type Codes

| Type | Description |
| --- | --- |
| 1 | PQ Bus (Load bus) - P and Q specified |
| 2 | PV Bus (Generator) - P and V specified |
| 3 | Slack Bus - V and angle specified |

## 2.3 Input Value Units

| Parameter | Unit | Notes |
| --- | --- | --- |
| Voltage (V_init) | per-unit | Typically 0.95-1.05 |
| Angle | degrees | Slack bus = 0° |
| Impedance (R, X) | per-unit | On system base |
| Susceptance (B) | per-unit | Total line charging |
| Power (P, Q) | MW, Mvar | Actual values |
| Length | km | For per-km calculations |
| Rating (rateA) | MVA | Thermal limit |

# 3. RUNNING THE SYSTEM

## 3.1 Prerequisites

Required Python Packages:
    pip install numpy

Optional (for PYPOWER conversion):
    pip install pypower

## 3.2 Interactive Mode

Step 1: Open command prompt/terminal
Step 2: Navigate to the folder containing the scripts
Step 3: Run:
    python run_power_flow.py

Step 4: Enter requested information:
    - Input file path (e.g., "E:\data\case300.py")
    - Variation strength (1=5%, 2=10%, 3=15%)
    - Number of samples (default: 200)

Step 5: Wait for completion
Step 6: Check output folder for results

## 3.3 Command Line Mode

Basic Usage:
    python run_power_flow.py -i <input_file> -v <variation> -n <samples>

Arguments:
    -i, --input      Path to input Python file
    -v, --variation  Variation strength in percent (5, 10, or 15)
    -n, --samples    Number of samples to generate

Examples:
    # Full parameters
    python run_power_flow.py -i "E:\data\case300.py" -v 10 -n 500

    # Default variation (10%), custom samples
    python run_power_flow.py -i case300.py -n 100

    # Short path
    python run_power_flow.py -i my_system.py -v 15

## 3.4 Variation Strength Explained

| Option | Strength | Voltage Range | Angle Range |
| --- | --- | --- | --- |
| 1 (5%) | Small | 0.95 - 1.05 pu | ±1.5° |
| 2 (10%) | Medium | 0.90 - 1.10 pu | ±3.0° |
| 3 (15%) | Large | 0.85 - 1.15 pu | ±4.5° |

Higher variation produces more diverse samples but may include unrealistic operating points.

# 4. OUTPUT SPECIFICATIONS

## 4.1 Output Files Generated

| File | Format | Contents |
| --- | --- | --- |
| *_ALL_DATA.csv | CSV | Complete input+output for all components |
| *_ALL_DATA.py | Python | Same data as CSV, loadable as module |
| *_OUTPUTS_ONLY.py | Python | Only calculation results (compact) |
| *_SUMMARY.txt | Text | System summary and key metrics |

## 4.2 Output Folder Structure

After running, outputs are saved in:
    <input_file_directory>/<input_filename>/
        ├── case300_var10_20240115_143000_ALL_DATA.csv
        ├── case300_var10_20240115_143000_ALL_DATA.py
        ├── case300_var10_20240115_143000_OUTPUTS_ONLY.py
        └── case300_var10_20240115_143000_SUMMARY.txt

Note: Folder is named after the input file.

## 4.3 Output Data Fields

BUS Outputs:

O_V_final_pu      - Final voltage magnitude (per-unit)
O_V_final_kV      - Final voltage magnitude (kV)
O_angle_deg       - Voltage angle (degrees)
O_voltage_status  - LOW/NORMAL/HIGH (based on 0.95-1.05 pu range)
O_angle_status    - NORMAL if |angle| < 30°, else HIGH

BRANCH Outputs:

O_P_fwd_MW        - Active power from→to (MW)
O_Q_fwd_Mvar      - Reactive power from→to (Mvar)
O_MVA_fwd_MVA     - Apparent power from→to (MVA)
O_P_rev_MW        - Active power to→from (MW)
O_Q_rev_Mvar      - Reactive power to→from (Mvar)
O_MVA_rev_MVA     - Apparent power to→from (MVA)
O_losses_MW       - Total losses (MW)
O_loading_pct     - Loading percentage
O_status          - NORMAL/MODERATE/HIGH/OVERLOADED

TRANSFORMER Additional Outputs:

O_tap_position    - Current tap ratio value
O_tap_step        - Integer tap step number
O_losses_MVA      - Losses in MVA (not just MW)

# 5. CONVERTING PYPOWER FILES

## 5.1 Single File Conversion

Run the converter:
    python frompypowerformatetothis.py

Select option 1 (Single File)
Enter the path to your PYPOWER .py file

Output: converted_<original_filename>.py in same folder

## 5.2 Batch Conversion

Run the converter:
    python frompypowerformatetothis.py

Select option 2 (Batch Mode)
Enter the folder path containing PYPOWER files

All .py files will be converted with "converted_" prefix

## 5.3 PYPOWER to Custom Format Mapping

| PYPOWER Field | Index | Custom Format |
| --- | --- | --- |
| bus[:,0] | 0 | bus_num |
| bus[:,1] | 1 | bus_type |
| bus[:,2] | 2 | P_demand → LOAD_DATA |
| bus[:,3] | 3 | Q_demand → LOAD_DATA |
| bus[:,4] | 4 | shunt_G |
| bus[:,5] | 5 | shunt_B → CAPACITOR/REACTOR |

# 6. TROUBLESHOOTING

Problem: Error: File not found

Solution: Check file path. Use forward slashes or escape backslashes: "E:\\data\\file.py"

Problem: Error: Could not find valid PYPOWER dictionary

Solution: The input file must have a function that returns a ppc dictionary

Problem: No valid samples generated

Solution: Reduce variation strength. Check input data for invalid impedances (R=0, X=0)

Problem: NaN values in output

Solution: Check for zero-impedance branches or disconnected buses

Problem: Loading > 100% everywhere

Solution: Check if rateA values are correct and not zero

Problem: Voltage values seem wrong

Solution: Verify base_kV values match actual system voltages

# 7. FREQUENTLY ASKED QUESTIONS

Q: What is "inverse" power flow?

A: Traditional power flow solves for voltages given powers. Inverse power flow calculates powers given voltages. This is useful for generating training data for machine learning models.

Q: Why do I need multiple samples?

A: Each sample represents a different operating point. More samples provide better statistical coverage of the operating envelope.

Q: Can I add my own components?

A: Yes. Modify input_reader.py to parse new data types and report_writer.py to output them. Do NOT modify power_flow_engine.py.

Q: What happens if rateA is 0?

A: Loading percentage is set to 0 (no limit). Losses are still calculated correctly.

Q: How accurate are the calculations?

A: The calculations use standard AC power flow equations with the π-model for lines and ideal transformer model. Accuracy depends on input data quality.

Q: Can I use this for real-time applications?

A: The current implementation is for offline analysis. For real-time use, you would need to integrate it with SCADA data and add convergence checking.