# POWER FLOW SYSTEM - QUICK REFERENCE

All 5 Files Summary in 3 Pages

# FILE 1: frompypowerformatetothis.py

Purpose: Converts PYPOWER format files to custom input format

Key Functions:

convert_pypower_to_custom(file_path)
  → Reads PYPOWER ppc dictionary
  → Extracts bus, gen, branch data
  → Separates lines (ratio=0) from transformers (ratio≠0)
  → Converts shunt_B to capacitor/reactor data
  → Writes converted_<filename>.py

Usage: Run script → Choose 1 (single) or 2 (batch) → Provide path

# FILE 2: input_reader.py

Purpose: Parses custom format input files and prepares data for engine

Key Functions:

read_from_python(file_path)
  → Loads .py file as module
  → Extracts all DATA variables into dictionaries
  → Returns structured data dict

convert_to_engine_format(input_data)
  → Converts to simple format for engine
  → Calculates line totals: R×length, X×length, B×length
  → Returns bus_data dict and branch_data list

Input Format Example:

BUS_DATA = [
    [1, "Bus_1", 3, 132, 1.05, 0.0, 0, 0, 1, 1, 1],
    [2, "Bus_2", 1, 132, 1.00, 0.0, 0, 0, 1, 1, 1]
]
LINE_DATA = [
    [1, "Line_1_2", 1, 2, 100, 0.0001, 0.0003, 0.00001, 200, 1, 1, 1, 1]
]

# FILE 3: power_flow_engine.py

Purpose: Core calculation engine - DO NOT MODIFY

Key Calculations:

build_admittance_matrix()
  Ybus[i][i] += y + jB          (line diagonal)
  Ybus[i][j] -= y               (line off-diagonal)
  Ybus[i][i] += y/a²           (transformer diagonal)
  Ybus[i][j] -= y/a            (transformer off-diagonal)

calculate_power(V_mag, V_angle)
  S = V × (Ybus × V)*
  P = real(S) × baseMVA
  Q = imag(S) × baseMVA

calculate_bidirectional_flows()
  Line:      S_fwd = Vi × [(Vi-Vj)×y + Vi×jB/2]*
  Xfmr:      S_fwd = Vi × [(Vi/a - Vj)×y]*
  Losses = S_fwd + S_rev

Sample Generation:

generate_sample(variation_strength)
  → Applies random ±variation to V_mag and V_angle
  → Clips voltage to 0.90-1.10 pu
  → Returns power injections and branch flows

# FILE 4: report_writer.py

Purpose: Generates output reports in multiple formats

Output Methods:

write_csv_all_data()      → *_ALL_DATA.csv
write_py_all_data()       → *_ALL_DATA.py  
write_py_output_only()    → *_OUTPUTS_ONLY.py
write_txt_summary()       → *_SUMMARY.txt

Output Includes:

• Bus: V_pu, V_kV, angle, status (LOW/NORMAL/HIGH)
• Generator: P_out, Q_out, V_actual, Q_status
• Load: P_supplied, Q_supplied, supply_status
• Line: P_fwd, Q_fwd, MVA_fwd, P_rev, Q_rev, MVA_rev, 
        losses, loss_per_km, loading_%
• Transformer: Same as line + tap_ratio, tap_step, loss_MVA
• Capacitor/Reactor: Q_injected/absorbed, V_actual
• System Summary: Total gen, load, losses, loss_%, voltages

# FILE 5: run_power_flow.py

Purpose: Main orchestrator - connects all components

Execution Modes:

Interactive:
  python run_power_flow.py
  → Prompts for file, variation, samples

Command Line:
  python run_power_flow.py -i file.py -v 10 -n 200
  -i: input file path
  -v: variation (5, 10, or 15 percent)
  -n: number of samples

Workflow:

1. Read input file (input_reader.py)
2. Convert to engine format (input_reader.py)
3. Initialize engine (power_flow_engine.py)
4. Run batch simulation (power_flow_engine.py)
5. Generate reports (report_writer.py)
6. Save to <input_filename>/ folder

# QUICK REFERENCE: DATA FLOW

| Stage | Input | Process | Output |
| --- | --- | --- | --- |
| 1. Convert | PYPOWER .py | frompypowerformatetothis.py | Custom .py |
| 2. Read | Custom .py | input_reader.py | Data dict |
| 3. Calculate | Data dict | power_flow_engine.py | Samples |
| 4. Report | Samples | report_writer.py | CSV/PY/TXT |
| 5. Run All | Command | run_power_flow.py | All files |

# KEY FORMULAS AT A GLANCE

Admittance:        y = 1/(R + jX) = (R - jX)/(R² + X²)
Power Injection:  S = V × (Ybus × V)*
Line Forward:     S_fwd = Vi × [(Vi-Vj)×y + Vi×jB/2]*
Xfmr Forward:     S_fwd = Vi × [(Vi/a - Vj)×y]*
Losses:           Loss = S_fwd + S_rev
Loading:          Loading_% = |S_fwd| × baseMVA / rateA × 100
Capacitor Q:      Q_out = Q_rated × V²
Voltage kV:       V_kV = V_pu × base_kV